# Object Speed Estimator Using Arduino Uno

## Project Overview

The Object Speed Estimator is an embedded systems project developed using an Arduino Uno, an HC-SR04 ultrasonic sensor, and a Flying Fish IR sensor. The system estimates the speed of a moving object by measuring the time it takes to travel between two sensors placed 28 cm apart.

The project was built through an iterative development process. AI was used to generate an initial implementation, which I then tested, modified, debugged, and refined over multiple iterations until the system behaved reliably and produced clean, consistent results.

# Objective

To design a real-time system capable of measuring the speed of an object using two different sensors while improving reliability, timing accuracy, and user interaction.

# Hardware Used

- Arduino Uno

- HC-SR04 Ultrasonic Sensor

- Flying Fish IR Obstacle Sensor

- Breadboard

- Jumper Wires

- USB Cable

- Arduino IDE

# System Layout

The two sensors were positioned in a straight line with a fixed distance of 28 cm between them.

- The ultrasonic sensor marks the starting point.

- The IR sensor marks the finishing point.

# How the System Works

### Step 1 — Idle

When powered on, the Arduino waits for an object to enter the ultrasonic sensor's detection zone.

The Serial Monitor displays:

Ready...

### Step 2 — Lap Start

When the object comes within 15 cm of the ultrasonic sensor:

- A new lap begins.

- The stopwatch starts.

- The Serial Monitor displays:

Lap 1 Started...

The elapsed time is then updated every 0.25 seconds until the object reaches the second sensor.

### Step 3 — Lap Finish

When the object reaches the IR sensor:

- The stopwatch stops.

- Total travel time is calculated.

- Speed is calculated using:

Speed = Distance ÷ Time

where the distance is fixed at 28 cm.

The Serial Monitor displays the final lap time and calculated speed.

### Step 4 — Reset

To avoid duplicate measurements, the system waits until the object leaves both sensors before allowing another lap to begin.

# Programming Approach

Rather than writing the entire program from scratch, I used AI to generate an initial version of the code. From there, I iteratively improved the implementation by testing the hardware, identifying issues, modifying the program logic, and refining the user interface.

My primary contribution was the engineering and debugging process rather than simply accepting the first generated solution. Through multiple rounds of testing and refinement, I transformed the initial code into a stable and reliable system.

# Challenges Encountered

Developing the project involved solving several practical problems.

### Continuous Triggering

Initially, the ultrasonic sensor repeatedly detected the same object, causing the system to continuously restart measurements.

This was solved by introducing a reset state that waits until the object completely leaves the sensing area before another lap can begin.

### Trigger Distance Calibration

The original trigger distance caused frequent unwanted measurements.

Different trigger distances were tested before settling on 15 cm, providing more consistent operation.

### Sensor Synchronization

Since two different sensors were responsible for starting and stopping the timer, synchronizing their behavior was essential.

The program logic was refined so that both sensors worked together as a single timing system.

### Serial Monitor Output

Early versions displayed excessive debugging information and flooded the Serial Monitor.

The interface was redesigned into a simple stopwatch-style display showing only the most relevant information.

### Testing

Several test runs were required using a toy car.

Each iteration helped improve the system's responsiveness, timing behavior, and overall reliability.

# Skills Demonstrated

- Arduino Programming

- Embedded Systems Development

- Sensor Integration

- Real-Time Timing using millis()

- Hardware Debugging

- State-Based Program Logic

- Sensor Calibration

- Iterative Software Development

- Problem Solving

- Human-AI Collaborative Development

- Technical Documentation

# Sample Output

OBJECT SPEED ESTIMATOR

Ready...

Lap 1 Started...

0.25 s

0.50 s

0.75 s

1.00 s

Lap 1

Final Time : 1.08 s

Speed : 25.93 cm/s

# Lessons Learned

This project reinforced that developing embedded systems is an iterative process. Small changes in sensor placement, trigger thresholds, or program logic can significantly affect system behavior.

I also learned that generating code is only the starting point. Building a reliable system required repeated experimentation, debugging, calibration, and refinement until the hardware and software worked together consistently.

# Future Improvements

Potential future enhancements include:

- Interrupt-based IR detection for faster objects

- OLED display

- SD card data logging

- Bluetooth connectivity

- Wireless dashboard

- Data visualization software

- Multiple speed statistics (minimum, maximum, and average)

# Conclusion

The Object Speed Estimator demonstrates how two different sensors can be combined to create a simple real-time measurement system.

While AI assisted in producing the initial code, the project was completed through continuous testing, debugging, calibration, and refinement. The experience strengthened my understanding of sensor integration, embedded programming concepts, and the importance of iterative engineering in developing reliable systems.





/*

==========================================

OBJECT SPEED ESTIMATOR

Arduino Uno

HC-SR04

TRIG -> D9

ECHO -> D10

Flying Fish IR

OUT -> D2

Sensor Distance : 28 cm

Trigger Distance: 15 cm

==========================================

*/

const int trigPin = 9;

const int echoPin = 10;

const int irPin = 2;

const float triggerDistance = 15.0;

const float sensorDistance = 28.0;

bool lapRunning = false;

bool waitingReset = false;

unsigned long startTime;

unsigned long lastUpdate;

int lap = 1;

//----------------------------------------

float getDistance() {

digitalWrite(trigPin, LOW);

delayMicroseconds(2);

digitalWrite(trigPin, HIGH);

delayMicroseconds(10);

digitalWrite(trigPin, LOW);

long duration = pulseIn(echoPin, HIGH, 30000);

if (duration == 0)

return 999;

return duration * 0.0343 / 2.0;

}

//----------------------------------------

void setup() {

pinMode(trigPin, OUTPUT);

pinMode(echoPin, INPUT);

pinMode(irPin, INPUT);

Serial.begin(9600);

Serial.println("OBJECT SPEED ESTIMATOR");

Serial.println();

Serial.println("Ready...");

Serial.println();

}

//----------------------------------------

void loop() {

float distance = getDistance();

// ---------------- Start Lap ----------------

if (!lapRunning && !waitingReset && distance <= triggerDistance) {

lapRunning = true;

startTime = millis();

lastUpdate = startTime;

Serial.print("Lap ");

Serial.print(lap);

Serial.println(" Started...");

Serial.println();

}

// ---------------- Stopwatch ----------------

if (lapRunning) {

unsigned long now = millis();

if (now - lastUpdate >= 250) {

lastUpdate += 250;

float elapsed = (lastUpdate - startTime) / 1000.0;

Serial.print(elapsed, 2);

Serial.println(" s");

}

// ---------------- Finish Lap ----------------

if (digitalRead(irPin) == LOW) {

float totalTime = (millis() - startTime) / 1000.0;

float speed = sensorDistance / totalTime;

Serial.println();

Serial.print("Lap ");

Serial.println(lap);

Serial.print("Final Time : ");

Serial.print(totalTime, 3);

Serial.println(" s");

Serial.print("Speed      : ");

Serial.print(speed, 2);

Serial.println(" cm/s");

Serial.println();

lap++;

lapRunning = false;

waitingReset = true;

}

}

// ---------------- Reset ----------------

if (waitingReset) {

if (digitalRead(irPin) == HIGH && distance > triggerDistance) {

waitingReset = false;

}

}

}